1. Project Overview

The Smart Waste Bin is an automated system designed to:

- Identify waste type (degradable vs non-degradable) using Raspberry Pi + Camera + AI model.

- Segregate waste into the correct bin using a differential mechanism controlled by ESP32 & motors.

- Detect mixed waste and apply an automatic processing fee.

- Detect heavy materials and request extra charges.

- Enable payment via UPI/App.

- Eject waste if the user refuses to pay.

- Monitor bin fill levels using sensors placed inside each bin.

- Send notifications to the mobile app when bins are full.

- Provide a 7-inch touchscreen interface for user interaction.

2.1 Major Components

Computing

- Raspberry Pi 4 / Pi 5

- 7-inch touch display

- Camera Module (Raspberry Pi Camera v3 / HQ Camera)

Control & Actuation

- ESP32 (motor control + sensors)

- High torque gear motors / differential plate system

- Servo/Stepper motors for plate positioning

- Weight sensor (load cell + HX711)

Sensors

- Ultrasonic / ToF Level Sensors for each bin

- IR break-beam sensor (detect object presence)

- Load cell for weight detection

- Emergency obstruction sensor

Communication

- UART/I2C between Raspberry Pi ↔ ESP32

- Wi-Fi for app communication

- MQTT / HTTP API as backend protocol

3. Functional Requirements

3.1 Waste Type Detection

- The camera captures an image of the waste when user inserts it.

- Raspberry Pi runs an AI classification model:

  - Degradable

  - Non-Degradable

  - *Mixed

  - High-Weight Object / Suspicious Material

AI Requirements

- Model: TensorFlow Lite / PyTorch Lite model

- Accuracy target: ≥ 90% for common waste items

- Inference speed: < 300 ms

3.2 Segregation Mechanism

- Controlled by ESP32.

- Differential rotating plate:

  - If waste = degradable → Rotate plate to Bin 1.

  - If waste = non-degradable → Rotate plate to Bin 2.

  - If mixed → Stop and request payment.

  - If high-weight → Stop and request extra fee.

Position Feedback

- Use high-resolution encoder or limit sensors to detect:

  - Neutral/home position

  - Degradable bin angle

  - Non-degradable bin angle

  - Eject bin angle

Movement Requirements

- Rotation time: ≤ 2 seconds

- Position accuracy: ±2°

3.3 Weight Measurement

- Load cell measures weight before dropping.

- Logic:

  - If weight < 500g → Normal process.

  - If 500g–3kg → Request handling fee.

  - If > 3kg → Treat as non-allowed item → eject.

Thresholds can be modified later.

3.4 Payment Handling

- Payment displayed on 7" screen.

- Allowed methods:

  - UPI QR

  - App-based online payment

- Raspberry Pi communicates with backend API:

  - Generate payment request

  - Verify payment status

If user pays

- Waste dropped into correct bin.

If user refuses / timeout

- ESP32 rotates plate to eject bin position and pushes waste out.

3.5 Bin Level Monitoring

Each bin includes:

- Ultrasonic or ToF level sensor

- Every 30–60 seconds, ESP32 measures bin level.

- Sends data to Pi → Cloud → App.

Alerts

- Bin > 80% full → “Near Full” alert

- Bin > 95% full → “Full – Stop Operation”

3.6 User Interface (7-inch Display)

UI screens:

- Welcome screen

- Instructions

- Waste scanning

- Identified category

- Payment required

- Payment success/failure

- Bin status information

- Maintenance/admin mode

3.7 Mobile App Requirements

- Real-time bin status (full/empty).

- Waste logs:

  - Time

  - Weight

  - Category

  - Payment info

- Notifications for:

  - Bin full

  - Camera failure

  - Motor jam

  - Sensor error

- Admin access to:

  - Reset system

  - View analytics

  - Set pricing

4. Communication Requirements

ESP32 ↔ Raspberry Pi

- UART for commands:

  - MOVE_NEUTRAL

  - MOVE_DEGRADABLE

  - MOVE_NONDEGRADABLE

  - MOVE_EJECT

  - READ_WEIGHT

  - READ_LEVEL_BIN1

  - READ_LEVEL_BIN2

Response format:
<CMD_RESPONSE>:<DATA>

Pi ↔ Cloud

- MQTT or REST API

- Data:

  - Waste logs

  - Bin level

  - Device health

  - Payment events

5. Mechanical Requirements

5.1 Differential Plate System

- Material: 3D printed PLA/ABS or aluminum.

- Size: Based on top opening.

- Required torque:

  - Load (max 3kg)

  - Plate radius

  - Use 30–50kg/cm geared DC motors OR NEMA17 stepper with gearbox.

5.2 Bin Design

- Two compartments:

  - Degradable

  - Non-degradable

- One eject chute.

5.3 Service Door

- Lockable front service panel:

  - Bin removal

  - Electronics access

6. Safety Requirements

- Auto-stop motor on:

  - Overcurrent

  - Motor jam

  - Obstruction detected

- Camera privacy: Only waste area captured.

- Fire safety: Material treated or metal casing.

7. Testing Requirements

AI Testing

- Test with 100+ waste item images.

- Measure accuracy.

Mechanical Testing

- Cycle test: 5,000 rotations min.

- Ensure no misalignment.

Load Test

- Verify weight detection at:

  - 100g, 500g, 1kg, 3kg

Payment Flow Testing

- Payment success

- Payment refused

- Timeout

Full Bin Test

- Sensor accuracy validation.

8. Deliverables for Technical Team

- Electrical schematics (ESP32 + sensors + motor drivers).

- PCB design (if needed).

- Raspberry Pi software:

  - AI model

  - UI application

  - Backend communication

- ESP32 firmware:

  - Motor control

  - Bin sensors

  - Level measurement

- Mechanical CAD models:

  - Plate

  - Gearbox

  - Bin frame

- API documentation for mobile app.